# Detecting emotions through EEG signals based on modified convolutional fuzzy neural network

## Introduction

- The research targets emotion recognition using EEG signals, a crucial topic for affective computing, healthcare, and human–computer interaction.

- Traditional emotion recognition relies on facial expressions or images, which are prone to concealment or manipulation, whereas EEG measures authentic, involuntary neural responses.

- However, EEG signals are complex, noisy, and high-dimensional, making robust, generalizable recognition challenging.

- Modern solutions seek advanced neural architectures with few parameters but high performance for real-time deployment.

## Related Works

- Early methods depended on traditional machine learning: KNN, SVM, Decision Trees, and Random Forests, leveraging hand-crafted features like fractal dimension and wavelet entropy. These methods are limited by the need for manual feature engineering and loss of relevant information at the preprocessing stage.

- Recent progress has been through Convolutional Neural Networks (CNN) and Long Short-Term Memory (LSTM) networks, often with feature fusion or attention mechanisms.

- Gamma frequency band and multi-channel feature fusion are repeatedly found to be most effective, but noise and inter-subject variability still limit accuracy in many approaches.

- Fuzzy neural networks (FNNs) that combine neural learning and fuzzy logic have emerged as viable tools for managing uncertainty and improving model interpretability and robustness.

## Methods

## Dataset

- DEAP dataset: 32 subjects (16 male, 16 female), each watched 40 emotion-evoking music videos.

- EEG recorded from 32 channels at 512 Hz and resampled to 128 Hz.

- The study focuses on classifying valence and arousal (binary, high/low).

## Signal Preprocessing and Feature Extraction

## 1. Down sampling and Filtering

- Raw EEG signals are down sampled from 512 Hz to 128 Hz to focus on content in the 0–48 Hz range and reduce redundancy, as emotion-linked EEG activity occurs primarily in low frequencies.

- Bandpass filtering (0–48 Hz) is applied to each channel to remove noise and physiological artifacts outside the relevant range.

- Non-EEG artifacts (baseline drifts, eye/muscle movement) are identified and removed after filtering to ensure clean input.

## 2. Channel Selection

- From the 32 available EEG channels, 14 are selected based on known emotional relevance—typically covering frontal, parietal, temporal, and occipital lobes to maximize emotion-specific neural activity captured.

## 3. Fast Fourier Transform (FFT)

- FFT is used to convert preprocessed time-domain signals into the frequency domain, yielding power spectra for each channel.

- Power features are computed for five key bands: theta (4–8 Hz), alpha (8–12 Hz), beta-low (12–16 Hz), beta-high (16–25 Hz), and gamma (25–45 Hz).

- The FFT window is varied; 32 samples found to be optimal for performance in experiments.

- Combined frequency-domain features from the 14 selected channels form the input for the neural network.

## Network Architecture: Modified CFNN

- 1D-CNN Layers: Two convolutional layers (with 64 and 32 filters, respectively) automatically extract temporal and spatial features from the sequential frequency-domain data.

- Max pooling: Reduces feature dimension and computational requirements while retaining crucial features.

- Fuzzification: CNN features are passed through a fuzzy layer utilizing Gaussian membership functions. This models uncertainty and manages noise, representing features in the fuzzy (continuous) domain.

- Batch normalization and dense layer: Normalize data, reduce overfitting, and optimize performance; a dense layer with two outputs for binary emotion classification.

- Defuzzification: Transforms fuzzy set activations back to crisp values for final decision-making, enabling robust and informative predictions.

- Total trainable parameters: 11,186 (ideal for computational efficiency and real-time applications).

## Experiments & Results

- Comparative experiments include SVM, KNN, Bi-LSTM, and 1D-CNN baselines.

- The proposed CFNN achieves:

  - Valence accuracy: 98.21%

  - Arousal accuracy: 98.08%

  - These results consistently outperform all conventional and contemporary deep learning baselines, especially in scenarios with high data variability and noise.

- Varying FFT window size (4–128) showed that sizes of 32 and above yielded best and most stable results.

## Discussion

- The study shows that robust preprocessing (clean signal acquisition, filtering, artifact removal), careful feature extraction using FFT, and a hybrid CNN–fuzzy logic architecture are key for high-accuracy emotion recognition from EEG.

- Gamma band and multi-channel data fusion are especially critical.

- The fuzzy post-processing layer after CNN extracts more discriminative, noise-resilient features.

- The approach is scalable and ready for real-time application, with clear avenues for further work on multiclass emotion recognition and generalization to other datasets.

## Conclusion

- Integration of advanced signal processing, FFT-based feature extraction, and a novel CFNN model allows nearly perfect binary emotion recognition on the DEAP EEG dataset.

- The method sets a new benchmark, highlighting the value of hybrid neural-fuzzy architectures and standard digital signal processing best practices.

- Future work will focus on multiclass emotion recognition and cross-dataset generalization.

Reference:

Ahmadzadeh Nobari Azar, Nasim, Nadire Cavus, Parvaneh Esmaili, Boran Sekeroglu, and Süleyman Aşır. "Detecting emotions through EEG signals based on modified convolutional fuzzy neural network." Scientific Reports 14, no. 1 (2024): 10371.

EEG-based emotion recognition systems; comprehensive study

### 1. Introduction and Significance

The paper establishes that EEG-based emotion recognition is a fundamental concept in artificial intelligence with major practical implications in emotional healthcare and human-computer interaction. It champions the use of physiological signals like EEG, arguing that they provide more objective and authentic results because they are spontaneously generated and not easily manipulated, unlike facial expressions or voice tone.

### 2. Emotion Modeling

The review discusses the theoretical basis for classifying emotions:

- Discrete Emotion Models: Categorizing emotions into fundamental states like anger, joy, or fear.

- Multi-Dimensional Models: The dominant framework is the 3D model of emotion space, based on three axes: Valence (pleasantness), Arousal (intensity), and Dominance (VAD) (control). The 3D model is preferred as it effectively differentiates similar emotions that occupy the same quadrant in the 2D (Valence-Arousal) model.

### 3. The Emotion Recognition (ER) System Framework

The core technical route for ER involves four fundamental stages: Signal Acquisition, Pre-processing, Feature Extraction, and Classification. The paper notes that the feature extraction step is crucial, as its quality directly impacts the performance of the recognition model.

#### Feature Extraction Methods

The review's major contribution is the comprehensive study of four types of EEG features:

- Frequency Domain Features (27% usage): Analyzing the power spectral density (PSD) of different brain waves (α,β,θ,δ).

- Time Domain Features (20% usage): Simple statistical attributes like amplitude, mean, and standard deviation.

- Time-Frequency Features: Combining time and frequency analyses, often using techniques like Wavelet Transform (WT) or Hilbert-Huang Transform (HHT).

- Nonlinear Features (7% usage): Analyzing the complexity and chaotic dynamics of the signal.

Crucially, the review highlights the growing use of Brain Network Features, which capture the complex and interdependent information exchange between multiple cerebral regions, an aspect ignored by traditional single-channel, time-frequency methods.

Datasets used:-

The review article discusses several prominent public datasets used for research in EEG-based emotion recognition.

SEED:

- Subjects: 15 (8 male / 7 female).

- Signals: EEG, Face Video, and Eye tracking.

- Stimuli: 15 video clips.

- Targets: Positive, Negative, and Neutral

- Query successful

- MPED:

  - Subjects: 23 (13 male / 10 female).

  - Signals: EEG, ECG, RSP (Respiration), and GSR.

  - Stimuli: 28 video clips.

  - Targets: Joy, Funny, Anger, Fear, Disgust, Sad, and Neutrality

### 4. Pattern Recognition (Classification)

The review summarizes current classification methods, devoting significant attention to modern approaches.

- Deep Learning (DL): DL is emphasized due to its capabilities in handling high-dimensional and non-linear EEG data and its ability to learn from raw EEG signals without needing extensive prior knowledge.

  - The most favored DL architectures identified are LSTM (50% usage) and CNN (36% usage), often with various derivatives.

- Classical Machine Learning (ML): Among traditional ML techniques, the Support Vector Machine (SVM) is the most prevalently utilized, accounting for 41% of usage in the summarized literature, valued for its effectiveness in high-dimensional space

### Challenges and Future Directions

The paper concludes by identifying the current challenges and outlining the future trajectory for the field:

- Methodology Optimization: A continued need to find the most optimal combination of features, channels, and classifiers.

- Generalization: Research must improve the resilience of models across different subjects and sessions, using techniques like domain adaptation and meta-learning.

- Key Identification: Identifying which specific brain regions and frequency bands are most closely associated with emotional states.

- Practical Application: The ultimate goal is to develop dependable, non-user-dependent wearable emotion recognition devices for seamless real-world use.

Electroencephalogram Based Emotion Recognition Using Hybrid Intelligent Method and Discrete Wavelet Transform

| Section | Simple Explanation |
| --- | --- |
| Title | EEG-Based Emotion Recognition Using a Hybrid Intelligent Method and Discrete Wavelet Transform |
| Authors | Duy Nguyen, Minh Tuan Nguyen, Kou Yamada |
| Published In | Applied Sciences (MDPI), 2025 |
| Main Goal | The paper aims to recognize people’s emotions using EEG brain signals. It combines digital signal processing (using wavelet transform) with artificial intelligence (CNN and boosting). |
| What They Did | - Broke down EEG signals into parts (called sub bands) using the Discrete Wavelet Transform (DWT). - Found useful features (energy, power, etc.) from these sub bands. - Used Genetic Algorithms (GA) to choose the best EEG channels and features. Then use a hybrid model (CNN + boosting) to classify emotions. |
| Dataset Used | EEG recordings (from the DEAP dataset or similar) with emotional labels — showing happy, sad, calm, or excited states. |
| DSP Technique | DWT was used to analyze signals at both times and frequency. It helps separate different brain rhythms (alpha, beta, theta, etc.) that change with emotions. |
| Feature Selection | The Genetic Algorithm was used twice: 1. To find which EEG electrodes (channels) give the best emotional information. 2. To select which features (out of 42) are most important. |
| Model Used | A hybrid model — CNN extracts features and Boosting improves accuracy. This combination performs better than using either alone. |
| Results (Accuracy, etc.) | - Valence (happy/sad) → 70.43% accuracy - Arousal (calm/excited) → 76.05% accuracy - Precision ≈ 70–75%, Recall ≈ 99%, F1-score ≈ 82–85% |
| Main Findings | - Emotions are mainly reflected in frontal brain regions. - Combining DWT, GA, CNN, and Boosting give better emotional detection. - Selecting the right channels and features improves results. |
| Strengths | - Uses smart channels and feature selection. - DWT captures emotion changes well. - Hybrid methods improve accuracy. |
| Limitations | - Still depends on machine learning (not pure DSP). - Processing time is long because of genetic algorithms. - Results may change for different people or datasets. |
| Future Work | Use faster or smarter algorithms and test real-time EEG devices for better emotional recognition. |

Reference

Nguyen, Duy, Minh Tuan Nguyen, and Kou Yamada. "Electroencephalogram Based Emotion Recognition Using Hybrid Intelligent Method and Discrete Wavelet Transform." Applied Sciences 15, no. 5 (2025): 2328.

EEG-based Emotion Recognition

### Introduction

- Aim: To make computers recognize human emotions using EEG (brain signals).

- Motivation: Facial or voice-based emotion detection can be faked or misunderstood — EEG gives a direct look at brain activity.

- Device used: BraInquiry PET EEG — a small EEG device with only 5 electrodes.

### Dataset Used

- IAPS – International Affective Picture System (set of emotion-labeled images).

- IADS – International Affective Digitized Sounds (set of emotion-labeled sounds).

- They selected 36 stimuli total:

- 12 visual (images)

- 12 auditory (sounds)

- 12 audiovisual (both together)

# Process after data collection

The recorded EEG signals were processed using digital signal processing techniques such as filtering to remove noise and extracting alpha and beta frequency bands, which are related to relaxation and alertness. Features like alpha power, beta power, and their ratio were then used to train a simple classifier (Fisher’s Discriminant Analysis) to identify the type of stimulus and the emotional state of the person in terms of valence (positive or negative) and arousal (calm or excited). The results showed that emotions could be recognized from EEG signals with fairly  high accuracy, proving that even a low-cost EEG setup can be used for emotion recognition.

### Role of DSP (Digital Signal Processing)

- DSP was essential for converting raw EEG signals into meaningful data:

  - Filtering – to remove noise and extract useful brainwave bands (alpha, beta).

  - Feature extraction – to compute power and ratios from frequency components using Fourier analysis.

# Limitations

- Very small dataset

- No personalized emotion labels

- Limited EEG setup

- Basic signal processing and classifier

- Experimental issues

### Conclusion

- EEG signals can successfully identify emotions (positive/negative, calm/excited).

- Even with only 5 electrodes, emotion recognition accuracy was high (>90%).

- This proves low-cost EEG + DSP techniques can enable emotion-aware computing.

- Future work: use more subjects, more electrodes, and advanced classifiers (like neural networks).

Reference

Oude Bos, Danny. "EEG-based emotion recognition." The influence of visual and auditory stimuli 56, no. 3 (2006): 1-17.